| PILOT CARD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Departure |  | Arrival | Shifting |  |  |  |  |  |
| VESSEL DETAILS |  |  |  |  |  |  |  |  |
| Vessel Name: AFRICAN CORMORANT |  |  |  |  | Port: MARSDEN PT. NZ |  |  |  |
| Call Sign: C6GC3 |  | MMSI.: 311 001 206 |  | Date: 12 APRIL 2025 |  |  |  |  |
| Flag: BAHAMAS |  | IMO No.: 9618006 |  | Year Built: 2013 |  |  |  |  |
| Deadweight: 24,199 t |  | Displacement: 33,696 t |  | GRT: 24725 |  |  |  | NRT: 13677 |
| Draft Forward: 6.83 m |  | Draft Amidships: 7.52 m |  | Draft Aft: 8.21 m |  |  |  |  |
| Air Draft: 36.02 m |  | Cargo Type: LOGS |  | Next Port: LANSHAN, CHINA |  |  |  |  |
| SHIP’S AGENT: |  |  |  |  |  |  |  |  |
| Length Overall: 179.99 mtrs |  | Breadth: 30.0 mtrs |  |  |  | Bulbous Bow? Yes No |  |  |
| Anchors Cleared and ready for use: YES / NO (1 shackle = 27.4 m/ 15 fathoms) |  |  |  |  |  |  |  |  |
| Port: 11 shackles |  | Starboard: 12 shackles |  |  |  |  | Stern: NO shackles |  |
| STEERING CHARACTERISTICS |  |  |  |  |  |  |  |  |
| Rudder | 1 SET No. | SCHILLING Type | Turning Circle: 400 m |  |  |  |  |  |
| Maximum Angle: _35 _ Degrees |  |  | Advance: 1879.29 m |  |  |  |  |  |
| Rudder angle for neutral effect: 35 Degrees |  |  | Transfer: _217.89__m |  |  |  |  |  |
| Hard over to hard over: _13 Sec with Both Motor |  |  | Seconds Radius: |  |  |  |  |  |
| Propellers | No. 1 | Type : Right Fix Propeller | Direction of turn: |  |  |  |  | Controllable pitch: FP |
| Thrusters | No | Type: N/A | Bow power: 6050 (kw/hp) |  |  |  |  | Stern power: 70% (kw/hp) |

KEY DISTANCES: (FILL UP DATA AS APPLICABLE)

B. 26.65 mtrs

B. 26.65 mtrs

E.

E.

C. 153.30m

C. 153.30m

F. 29.23m

F. 29.23m

H. 44.23m

H. 44.23m

A. 30m

A. 30m

BRIDGE

BRIDGE

G. 9.5m

G. 9.5m

D. (Loaded 111.00 m) (Ballast 103.04 m)

D. (Loaded 111.00 m) (Ballast 103.04 m)

| A | Breadth : 30 m | E | Distance: |
| --- | --- | --- | --- |
| B | Dist from Bridge to Aft: 26.65 m | F | Air Draught: 29.23 m |
| C | Dist from Bridge to Fwd: 153.30 m | G | Draught: 9.5 m |
| D | Parallel W/L:(Loaded 111.00 m) (Ballast 103.04 m) | H | Height Keel to Mast: 44.23 m |

| NAVIGATION AIDS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Compass system operational and ready |  | Yes / No | Constant Gyro error + / |  | degrees |
| Electronic position fixing system operational |  | Yes / No | Type |  |  |
| Echo Sounder operational and ready |  | Yes / No | Speed log operational and ready |  | Yes / No |
| ECDIS updated and Ready |  | Yes / No | Doppler |  | Yes / No |
| Steering Gear No. of power units in use |  | 1 or 2 | If Doppler log: Water speed / Ground speed / Dual Axis |  |  |
| Radars operational and ready | 3 cm | Yes / No | Indicators-operational | RPM / Pitch | Yes / No |
|  | 10 cm | Yes / No |  | Rudder Angle | Yes / No |
|  | ARPA | Yes / No |  | Rate of Turn | Yes / No |
| Navigation light Operational |  | Yes / No | Mooring winches and lines |  | Yes / No |
| Manoeuvring Data made available to Pilot |  | Yes / No | VHF operational and ready |  | Yes / No |
| Squat Data made available to Pilot |  | Yes / No | Whistle(s) Tested and Operational |  | Yes / No |

ENGINE:

| Type of engine:__MAN YMD 5S50ME-B9.2__Maximum power: 6050 (kw) Power with EPL __________ (kw) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AHEAD |  |  |  |  | ASTERN |  |  |  |  |  |  |
|  | RPM | LOADED SPEED (kts) |  | BALLAST SPEED (kts) |  |  |  | RPM |  | LOADED SPEED (kts) | BALLAST SPEED (kts) |
| Dead Slow | 25 | 3.50 |  | 3.70 | Dead Slow |  |  | 25 |  | - | - |
| Slow | 40 | 6.75 |  | 6.91 | Slow |  |  | 40 |  | - | - |
| Half | 62 | 9.97 |  | 10.11 | Half |  |  | 65 |  | - | - |
| Full | 72 | 11.45 |  | 11.56 | Full |  |  | 70 |  | - | - |
| Engine Critical rpm: 48.5 |  | Min.steering speed 2.5 KTS |  |  |  |  |  | Max. no. of consecutive starts: 12 |  |  |  |
| Time full ahead to full astern: _335 (sec) |  |  |  |  |  |  | Time limit astern: 20 (min) |  |  |  |  |
| Bridge control / ER control |  |  | Engine telegraphs operational Yes /No |  |  |  |  |  | Engines tested Ahead/Astern: Yes/No |  |  |
| MASTER PILOT INFORMATION EXCHANGE |  |  |  |  |  |  |  |  |  |  |  |
| - EQUIPMENT OPERATIONAL DETAILS / DEFECTS AFFECTING MANOEUVRABILITY: |  |  |  |  |  |  |  |  |  |  |  |
| - EPL / SHAPoli DETAILS (IF APPLICABLE) & OTHER IMPORTANT INFORMATION |  |  |  |  |  |  |  |  |  |  |  |
| Is the vessel equipped with an engine or shaft power limiter? YES / NO (tick one) |  |  |  |  |  |  |  |  |  |  |  |
| Type of EPL system (Mechanical or software based): N/A |  |  |  |  |  |  |  |  |  |  |  |
| Power with EPL: kW |  |  |  |  |  | Power without EPL: kW |  |  |  |  |  |
| Time required to override EPL: |  |  |  |  |  |  |  |  |  |  |  |
| Other important information (Windage Area, Location of AIS antenna, SWL of Bollards etc): |  |  |  |  |  |  |  |  |  |  |  |

(PILOTS ARE REQUESTED TO FILL IN / ADVISE THIS INFORMATION TO MASTER)

- WEATHER CONDITIONS

- Wind conditions in harbour: Direction _________ Speed ____ knots

- Tide: state - EBB / Flood / Slack* Height of tide: above chart datum ______ ______

- Local forecast: _____________________

- LOCAL REGULATIONS

- Max. Allowable Draft: _____________ mtrs.

- Min UKC required: on passage ______ at berth ___ mtrs. _______

- VHF Channel: River _________ Harbour______ Port Control________ Tugs_________

- VTS Reporting Points: ____________

- Speed Restrictions (if any): _________________________

- PASSAGE PLAN

- Route to follow giving main Waypoints: A/P PASSAGE PLAN

- Abort points and Contingency Anchorages: MARKED ON ECDIS

- Speed requirements at different legs: ___________________

- Traffic status: ________________________________________________________________

- If anchor stations to be called. Yes/No*. If ‘Yes’ approx. position anchor stations required: ____________________________________

- Special lights / shapes to be exhibited by the vessel: ________________

- Navigational Hazards, Navigational Warnings, Recent changes of Navigational Marks or charted depths: ________________________________________________________

- Passage plan, ECDIS setting and Squat Effect discussed with Pilot. Yes/No

- BERTH AND TUG DETAILS

- Intended berth ___ ________. Side alongside: ___________

- Estimated transit time: ____ __________________________________________

- Does pilot require to check any documents? Yes / No*

- Tug Rendezvous Position _____ _______. Number of Tugs _________. Tug securing arrangement: (F) ______________________ (A) _______________________ Bollard Pull of Tug: _____________________________________________________

- Ship’s gangway/shore gangway*. If ship’s gangway can be rigged prior berthing? Yes / No*

- Berthing Plan: First Line (FWD) __________________________ (AFT) _________________________ Final Tie Up (FWD) ________________________ (AFT) _________________________ First Line ashore by heaving line / mooring boat*

- CHANGE OF PILOT BOARDING INSTRUCTIONS

1. Position where pilot expected to change: ___________________________________________ 2. If Pilot ladder or Accom. Ladder required: _____________ Side required ____________. Height above waterline: ________. Manropes required: Yes / No. 3. Approach Course and Speed at Boarding Station: _____________________________________ 4. If any port official boarding while manoeuvring? Yes/No*. If ‘Yes’ position where expected to board: ___________________________________________

| MASTER |  | PILOT |  |
| --- | --- | --- | --- |
| NAME | CAPT. JHASON L. DE CASTRO | NAME |  |
| SIGNATURE |  | SIGNATURE |  |
| DATE | 12 APRIL 2025 | DATE |  |