VESSEL DETAILS : Port : Marsden Point, NZ (Arrival) Ship’s Name: HUPEH Flag: SINGAPORE Date: 26.05.2025 Call Sign: 9V2463 IMO No.: _ 9714264 Year Built: _2015_ Deadweight: 26423 MT Displacement: 36902 MT Gross Tonnage: __24785______ Net Tonnage: _____13022________ Draft Forward: ___8.00_____ m Draft Aft: __8.20____ m Freeboard: _6.90__m AIR DRAFT: ___35.99___ m Cargo Type : Logs Last / Next Port : New Plymouth, NZ / Lanshan, China

SHIP’S PARTICULARS: Length Overall: 179.99 m Breadth: 30.0 m Bulbous Bow?  Yes ☑ No Anchor Chain : Port 11 shackles Starboard 12 shackles (1 shackle = 27.4 m/ 15 fathoms)

STEERING PARTICULARS: Type of rudder: SEMI-SPADE Maximum Angle: 35 degrees Rudder angle for neutral effect: 0.2 degrees Hard over to hard over: 13.7 seconds Thruster: Bow __________kW (____N/A____HP) Stern_______ kW (_____N/A___HP)

KEY DISTANCES: (FILL UP DATA AS APPLICABLE)

26.69

Distance

Distance

153.30

BRIDGE

Breadth

30 m

Parallel (water line)

Parallel (water line)

Loaded    113	   m (T=9.5m)

Loaded     111   m ( T=10.5m)

Ballast	     103	   m (T=5.97m)

36.99       M

Height

44.2m

Height

□Left     ☑Right

| CHECK OPERATIONAL: |  |
| --- | --- |
| ☑ Anchors Cleared Away ☑ Yes No ☑ Whistle ☑ Radar 3CM ☑ 10CM ☑ ARPA ☑ Echo Sounder ☑ AIS ☑ Pilot Plug ☑ ECDIS ☑ Steering lights | ☑ Steering Gear No. of units operating :2 UNIT ☑ Compass System Gyro error : 0.0 ☑ RPM Indicator ☑ Engine Telegraphs ☑ Rudder Indicator ☑ VHF ☑ GPS ☑ Speed log - Doppler YES/NO ☑ Water speed ☑ Ground speed |

TYPE OF ENGINE: Maximum Power: 6050 kW Engine Critical RPM _____51-63__________ EPL or governor installed: NO Maximum available power as allowed by EPL or governor installed (if applicable) ___N/A___

| ENGINE ORDER | RPM RPM | SPEED (KNOTS) | SPEED (KNOTS) |
| --- | --- | --- | --- |
|  |  | LOADED | BALLAST |
| Full ahead | 68 | 10.5 | 11.5 |
| Half ahead | 48 | 7.6 | 8.7 |
| Slow ahead | 40 | 6.4 | 7.6 |
| Dead slow ahead | 30 | 4.9 | 6.1 |
| Dead slow astern | 30 | Time limit astern _30 RPM / 10_ min Full ahead to full astern ___182________ seconds Maximum number of consecutive starts __12_____ Minimum RPM __4.9__ Kts Astern power __70_____% ahead |  |
| Slow astern | 40 |  |  |
| Half astern | 48 |  |  |
| Full astern | 68 |  |  |

- END -